CHAPTER 8

# Evaporation — Theory, Equipment, and Pharmaceutical Applications

| CO No. | Course Outcome | Bloom's Level |
| --- | --- | --- |
| CO3 | Compare and evaluate different thermal processes such as drying, evaporation, distillation, and heat transfer based on operational efficiency. | L4 |
| CO2 | Identify and describe construction, working, and utility of evaporation equipment. | L2, L3 |

## 8.1 Learning Objectives

After completing this chapter, the student will be able to:

- Define evaporation and distinguish it from drying and distillation

- Explain Boiling Point Rise (BPR) and its significance in evaporator design

- Describe the concept of Economy and its calculation for single and multiple effect evaporators

- Explain the construction, working, advantages, disadvantages, and pharmaceutical applications of natural circulation, forced circulation, and multiple effect evaporators

- Describe falling film and climbing film evaporators and compare their characteristics

- Explain vacuum evaporation and its importance for thermolabile pharmaceutical solutions

## 8.2 Definition and Objectives of Evaporation

Evaporation is the process of concentrating a solution by vaporising and removing the volatile solvent (usually water) from a non-volatile solute (the dissolved drug or extract), leaving a more concentrated liquid solution or, in extreme cases, a solid. Evaporation is distinguished from:

- Drying — evaporation removes solvent from a solution (liquid); drying removes moisture from a solid or semi-solid material

- Distillation — distillation collects and uses the vapour phase; evaporation discards the vapour and uses the concentrated liquid

Objectives of evaporation in pharmaceutical manufacturing:

- Concentration of herbal extracts and tinctures before further processing (soft extract, dry extract preparation)

- Concentration of syrup solutions — sugar syrups, medicated syrups

- Concentration of API solutions from extraction processes before crystallisation

- Pre-concentration of fermentation broths before isolation of antibiotics

- Preparation of parenteral nutrition solutions and concentrated electrolyte solutions

- Recovery of solvents (by evaporation + condensation = distillation) in solvent-based pharmaceutical processes

## 8.3 Theory of Evaporation

### 8.3.1 Boiling Point Rise (BPR)

When a non-volatile solute is dissolved in water (or any solvent), the boiling point of the solution is elevated above the boiling point of pure solvent — this elevation is called Boiling Point Rise (BPR). BPR increases as the solution becomes more concentrated during evaporation.

BPR = i × Kb × m

Where: i = van't Hoff factor (number of particles per formula unit); Kb = ebullioscopic constant of solvent (0.512 °C·kg/mol for water); m = molality of solution (mol solute/kg solvent). In practical evaporator design, BPR means the temperature driving force (steam temperature − boiling point of solution) is reduced compared to pure water — reducing heat transfer efficiency and evaporation rate.

### 8.3.2 Economy — Evaporator Efficiency

Economy is defined as the kg of water evaporated per kg of steam supplied. It is the primary measure of evaporator thermal efficiency:

Economy = kg water evaporated / kg steam consumed

A single-effect evaporator has Economy ≈ 0.8–0.9 (slightly less than 1 because some heat is lost to the surroundings and to heating the feed to boiling point). Multiple-effect evaporation dramatically improves economy by reusing the vapour from one effect as the heating medium of the next:

| Configuration | Theoretical Economy | Practical Economy | Capital Cost | Running Cost |
| --- | --- | --- | --- | --- |
| Single effect | ≈ 1.0 | 0.8–0.9 | Lowest | Highest (most steam) |
| Double effect (2-effect) | ≈ 2.0 | 1.5–1.8 | Medium | Medium |
| Triple effect (3-effect) | ≈ 3.0 | 2.0–2.5 | Higher | Lower |
| Quadruple effect (4-effect) | ≈ 4.0 | 2.5–3.5 | High | Lower still |
| Vapour recompression (TVR/MVR) | 5–10+ | Equivalent to 5-10 effects | High (compressor) | Very low (electricity only) |

### 8.3.3 Heat Transfer in Evaporators

Q = U × A × ΔT_lm

Where: Q = heat transferred (W); U = overall heat transfer coefficient (W/m²K); A = heat transfer area (m²); ΔT_lm = log mean temperature difference between steam (or hot fluid) and boiling solution (K). For evaporators: ΔT = T_steam − T_boiling solution. BPR reduces ΔT, requiring larger area A or higher U to maintain the same evaporation rate Q.

## 8.4 Natural Circulation Evaporators

### 8.4.1 Short-Tube (Calandria) Evaporator

| [ QR ] SCAN TO VIEW | 📱 Short-Tube (Calandria) Evaporator — Natural Circulation Interactive Animation + 5-Min Concept Lesson + Practice MCQs Steam chest, tube bundle, natural convection circulation, liquid-vapour separation, pharmaceutical extract concentration [ QR code will be inserted here after GitHub setup ] |
| --- | --- |

Principle: A vertical bundle of short tubes (calandria) is heated externally by steam condensing on the shell side. Solution inside the tubes is heated, boils, and the vapour-liquid mixture rises through the tubes. Vapour disengages in the vapour space above; liquid falls back through a central downcomer and recirculates. Circulation is driven by density difference — no pump required.

| Component | Description |
| --- | --- |
| Steam chest (calandria) | Vertical shell containing tube bundle; steam condensing on tube outer surface; steam inlet, condensate outlet |
| Tube bundle | Short vertical tubes (typically 0.9–1.8 m long, 50–75 mm diameter); solution boils inside tubes; steam heats outside |
| Central downcomer | Large central unheated tube or annular space; liquid descends to recirculate; creates the natural convection loop |
| Vapour space | Large cylindrical space above tube bundle; vapour separates from entrained liquid; vapour exits at top |
| Liquid level controls | Maintain solution level above tube bundle top; feed inlet and concentrated product outlet |
| Entrainment separator | Baffles or mesh separator above vapour space; prevents liquid droplets being carried out with vapour (entrainment loss) |

Applications: Concentration of herbal tinctures, sugar syrups, glycerol solutions, electrolyte solutions. Not suitable for: viscous solutions (poor circulation); heat-sensitive solutions (high temperature at tube wall); fouling solutions.

### 8.4.2 Long-Tube Vertical Evaporator — Climbing Film

| [ QR ] SCAN TO VIEW | 📱 Climbing Film Evaporator Interactive Animation + 5-Min Concept Lesson + Practice MCQs Rising film mechanism, flash evaporation, thin film formation, pharmaceutical viscous extract concentration [ QR code will be inserted here after GitHub setup ] |
| --- | --- |

Principle: Feed enters at the bottom of long vertical tubes (3–10 m long, 25–50 mm diameter) heated by steam on the shell side. As liquid climbs up the tubes, it heats and begins to boil. The vapour formed inside the tubes carries the liquid upward as a thin film — climbing film. Liquid and vapour exit together at the top and are separated. Once-through operation — no recirculation.

| Advantages | Disadvantages |
| --- | --- |
| Low residence time (seconds) — minimal thermal degradation; suitable for thermolabile solutions | Not suitable for viscous liquids or solutions that foam |
| High heat transfer coefficient — thin film, high velocity | Requires minimum liquid flow rate for film formation — turndown ratio limited |
| Handles dilute solutions efficiently | Cannot achieve high concentration ratios in single pass — multiple passes needed |
| No liquid recirculation — low hold-up volume |

### 8.4.3 Falling Film Evaporator

| [ QR ] SCAN TO VIEW | 📱 Falling Film Evaporator Interactive Animation + 5-Min Concept Lesson + Practice MCQs Liquid distribution at top, falling film inside tubes, gentle evaporation, pharmaceutical dairy and API solution concentration [ QR code will be inserted here after GitHub setup ] |
| --- | --- |

Principle: Feed enters at the TOP of long vertical tubes. A liquid distributor ensures uniform feed distribution across all tubes. Liquid flows DOWNWARD as a thin film on the inner tube wall — gravity-assisted. Steam heats the outer tube surface. Vapour forms at the liquid surface and flows downward (co-current) or upward (counter-current) to be separated. Low temperature difference and short residence time make this ideal for heat-sensitive materials.

| Feature | Climbing Film | Falling Film |
| --- | --- | --- |
| Liquid flow direction | Upward (vapour carries liquid) | Downward (gravity) |
| Driving force | Vapour velocity carrying liquid | Gravity |
| Residence time | Low (seconds) | Very low (< 1 second) |
| Viscosity handling | Moderate | Low to moderate |
| Heat sensitivity | Good | Excellent — preferred for thermolabile |
| Pharmaceutical use | Herbal extract concentration | Fruit juice, dairy, heat-sensitive API solutions, WFI stills |

## 8.5 Forced Circulation Evaporator

| [ QR ] SCAN TO VIEW | 📱 Forced Circulation Evaporator Interactive Animation + 5-Min Concept Lesson + Practice MCQs External heat exchanger, circulating pump, flash chamber, viscous pharmaceutical solution concentration, fouling prevention [ QR code will be inserted here after GitHub setup ] |
| --- | --- |

Principle: In forced circulation evaporators, a pump continuously circulates the solution at high velocity through an external heat exchanger (shell-and-tube type). The solution is heated under positive pressure to prevent boiling in the heat exchanger (flash prevention). The hot pressurised liquid then enters a flash chamber where the pressure is reduced — the solution flashes and evaporates rapidly. The concentrated solution is recycled by the pump; vapour exits the flash chamber to a condenser.

| Component | Description |
| --- | --- |
| Circulation pump | Centrifugal pump; circulates liquid at high velocity (1.5–3 m/s); high velocity prevents fouling on heat exchanger tubes; provides positive pressure |
| External heat exchanger | Shell-and-tube heat exchanger; steam on shell side; solution at high pressure on tube side — does not boil in exchanger; rapid heating |
| Flash chamber (separator) | Vertical cylindrical vessel; liquid enters under reduced pressure — flashes and boils; vapour-liquid separation; liquid level controlled by pump speed |
| Vapour outlet | To condenser or next effect; entrainment separator prevents liquid carryover |
| Heating steam and condensate | Steam supply to heat exchanger shell; condensate removed automatically |

| Advantages | Disadvantages |
| --- | --- |
| Handles viscous solutions (up to 100,000 mPa·s) — forced flow prevents stagnation | Higher energy consumption — pump adds significant power consumption |
| Prevents fouling — high liquid velocity maintains turbulent flow at heat exchanger surface | More complex — pump, heat exchanger, and flash chamber all require maintenance |
| Can concentrate to high solids content | Higher capital cost than natural circulation |
| Suitable for crystallising solutions — slurry circulated without blockage | Temperature-sensitive solutions may be damaged in long residence time at elevated temperature |

- Applications: Concentration of viscous herbal extracts; concentration of sugar solutions to high Brix; crystallising evaporators for API salts; evaporation of solutions prone to fouling (starch, protein-containing solutions)

## 8.6 Multiple Effect Evaporation

| [ QR ] SCAN TO VIEW | 📱 Multiple Effect Evaporator — Triple Effect System Interactive Animation + 5-Min Concept Lesson + Practice MCQs Steam flow through 3 effects, vapour reuse mechanism, economy calculation, pharmaceutical large-scale solvent removal [ QR code will be inserted here after GitHub setup ] |
| --- | --- |

Multiple effect evaporation dramatically reduces steam consumption by reusing the vapour from each evaporator as the heating steam for the next. In a triple-effect evaporator:

- Effect 1: Live steam (high pressure) heats the solution; vapour produced at intermediate pressure

- Effect 2: Vapour from Effect 1 (now the heating medium) heats solution in Effect 2 at lower pressure; vapour produced at lower pressure

- Effect 3: Vapour from Effect 2 heats solution in Effect 3 at lowest pressure (or vacuum); vapour from Effect 3 goes to condenser

- Economy ≈ 2.5 for triple effect (theoretical 3.0) — approximately 3 kg water evaporated per kg live steam

Feed arrangements in multiple effect evaporation:

| Arrangement | Description | Advantage | Pharmaceutical Use |
| --- | --- | --- | --- |
| Forward feed | Feed enters Effect 1 (highest temperature); concentrated product leaves Effect 3 (lowest temperature) | Simple; no pumps between effects; most concentrated solution at lowest temperature — benefits thermolabile solutions | Thermolabile herbal extracts; heat-sensitive API solutions |
| Backward feed | Feed enters Effect 3 (lowest temperature, highest vacuum); product leaves Effect 1 (highest temperature) | Most concentrated solution at highest temperature — better viscosity for pumping; better economy for cold feeds | Viscous syrups; solutions that crystallise at low temperature |
| Mixed feed | Combination of forward and backward feed | Balances advantages of both | Large-scale evaporation plants where different products processed |

## 8.7 Vacuum Evaporation

Vacuum evaporation involves operating the evaporator under reduced pressure (vacuum), which lowers the boiling point of the solution. This is essential for:

- Thermolabile pharmaceutical solutions — vitamins, enzymes, antibiotics, herbal extracts containing volatile bioactive compounds — would degrade at the normal boiling point of water (100°C) but are stable at 40–60°C (achievable under vacuum of 60–100 mbar)

- Concentration to high solids content — under vacuum, solutions can be concentrated beyond 50% solids without charring or degradation

- Recovery of solvents — organic solvents (ethanol, methanol, acetone) have low boiling points and can be recovered efficiently under vacuum

| Vacuum Level (mbar) | Boiling Point of Water (°C) | Application |
| --- | --- | --- |
| 1013 (atmospheric) | 100°C | Non-thermolabile solutions; industrial evaporation |
| 100–200 mbar | 60–70°C | Mildly thermolabile solutions; herbal extracts |
| 30–60 mbar | 40–50°C | Moderately thermolabile — vitamins, sugar-sensitive solutions |
| < 20 mbar | < 40°C | Highly thermolabile — enzymes, biologics, heat-sensitive APIs |

## Industry Box 8.1 — Triple Effect Evaporation of Neem Leaf Extract at Himalaya Drug Company

Product: Neem (Azadirachta indica) leaf extract concentrate | Company: Himalaya Drug Company, Bangalore | Equipment: Triple effect falling film evaporator

Fresh neem leaves are extracted with 60% ethanol:water in percolators. The resulting 5% w/v extract contains azadirachtin, nimbin, and nimbinin as primary bioactives. To prepare soft extract (dry extract precursor), the extract must be concentrated from 5% to 40% total solids — an 8-fold concentration.

Challenge: Azadirachtin and related limonoids are heat-sensitive — significant degradation at temperatures above 50°C. Conventional single-effect evaporation at atmospheric pressure (100°C) destroys > 60% of bioactive content within 20 minutes.

Solution — Triple effect falling film evaporator under vacuum:

Effect 1: Feed (5% solids) enters at top of first evaporator; vacuum 80 mbar; boiling point 47°C; concentrate exits at 12% solids

Effect 2: Partially concentrated solution enters Effect 2; vacuum 50 mbar; boiling point 38°C; exits at 25% solids

Effect 3: Solution enters Effect 3; vacuum 25 mbar; boiling point 30°C; exits at 40% solids — SOFT EXTRACT

Economy achieved: 2.4 kg water evaporated per kg steam (vs. 0.85 for single effect) — saves 65% on steam costs.

Engineering lesson: The combination of falling film design (very short liquid residence time — < 2 seconds per effect) + vacuum evaporation (boiling at 30–47°C) + triple effect (economy 2.4) achieves: maximum bioactive preservation; minimum energy consumption; acceptable throughput. This is an engineering solution to a biological problem — preserving heat-sensitive plant compounds during concentration.

## Worked Example 8.1 — Economy and Evaporation Calculations

Problem: A single-effect evaporator concentrates 1000 kg/hr of a 5% w/w herbal extract solution to 25% w/w. Steam is supplied at 110°C (latent heat λ_s = 2230 kJ/kg). Feed enters at 25°C. Boiling point of solution = 105°C (BPR = 5°C under atmospheric pressure). Latent heat of solution at 105°C = 2250 kJ/kg. Calculate: (a) water evaporation rate, (b) steam consumption rate, (c) economy.

Step 1 — Mass balance (water evaporation rate):

Feed: 1000 kg/hr at 5% solids → solids = 1000 × 0.05 = 50 kg/hr

Product: at 25% solids → total product = 50 / 0.25 = 200 kg/hr

Water evaporated (W) = Feed − Product = 1000 − 200 = 800 kg/hr

Step 2 — Heat balance (steam consumption):

Heat required = Heat to raise feed from 25°C to 105°C + Heat for evaporation

Q_sensible = Feed × Cp × (T_boil − T_feed) = 1000 × 4.18 × (105−25) = 1000 × 4.18 × 80 = 334,400 kJ/hr

Q_latent = W × λ_solution = 800 × 2250 = 1,800,000 kJ/hr

Q_total = 334,400 + 1,800,000 = 2,134,400 kJ/hr

Steam consumption = Q_total / λ_steam = 2,134,400 / 2230 = 957 kg/hr

Step 3 — Economy:

Economy = Water evaporated / Steam consumed = 800 / 957 = 0.84 kg water/kg steam

Interpretation: Economy = 0.84 — typical single-effect economy. If upgraded to triple effect, economy ≈ 0.84 × 3 = 2.5 kg water/kg steam — saving approximately 67% of steam costs at the expense of higher capital investment in additional effects.

## Key Points — Chapter 8

1. Evaporation concentrates solutions by vaporising solvent (usually water) — distinguished from drying (removes moisture from solids) and distillation (collects the vapour).

2. Boiling Point Rise (BPR) = i × Kb × m. Solutions boil at higher temperature than pure solvent — BPR reduces the effective temperature driving force in evaporators.

3. Economy = kg water evaporated / kg steam consumed. Single effect ≈ 0.85; double effect ≈ 1.6; triple effect ≈ 2.5. Multiple effects improve economy by reusing vapour as heating medium.

4. Short-tube (calandria) evaporator: natural circulation through vertical tubes; steam on shell side; downcomer for recirculation; simple; limited to low-viscosity, non-fouling solutions.

5. Climbing film evaporator: liquid rises through long tubes as thin film; vapour forms and carries liquid upward; low residence time; good for dilute, thermolabile solutions.

6. Falling film evaporator: liquid falls downward through tubes as thin film; gravity driven; very short residence time (< 1 second); BEST for thermolabile pharmaceutical solutions — vitamins, enzymes, herbal extracts.

7. Forced circulation evaporator: pump drives liquid through external heat exchanger at high velocity; liquid flashes in separate chamber; handles viscous and fouling solutions; highest throughput.

8. Multiple effect evaporation: vapour from each effect used as heating medium for next; forward feed preferred for thermolabile solutions (most concentrated at lowest temperature).

9. Vacuum evaporation: reduces boiling point (100 mbar → 60°C; 25 mbar → 30°C); essential for thermolabile solutions; enables concentration of heat-sensitive herbal extracts, vitamins, antibiotics.

10. Heat transfer equation: Q = U × A × ΔT_lm. Increasing U (turbulence), A (area), or ΔT (temperature difference) increases evaporation rate. BPR and fouling reduce effective ΔT.

11. Falling film evaporators are widely used in pharmaceutical industry for: herbal extract concentration; WFI still (distillation of purified water); fruit juice concentration; thermolabile API recovery.

12. Economy calculation: water evaporated = Feed − Product; heat balance gives steam consumption; Economy = water evaporated / steam consumed. Typical single-effect: 0.84.

## Multiple Choice Questions — Chapter 8

(15 Questions — 1 mark each)

Q1. Economy of an evaporator is defined as:

- (A) Cost of evaporation per litre of product

- (B) kg of water evaporated per kg of steam supplied

- (C) Heat transfer area per unit evaporation rate

- (D) Ratio of feed flow rate to product flow rate

Answer: (B) | Economy = kg water evaporated / kg steam consumed. It measures thermal efficiency. Single effect ≈ 0.85 (slightly less than 1 due to heat losses and sensible heat for heating feed). Multiple effects dramatically improve economy by reusing vapour as heating medium — triple effect achieves ≈ 2.5.

Q2. Boiling Point Rise (BPR) in evaporator design is significant because:

- (A) It increases steam pressure requirements

- (B) It reduces the effective temperature driving force (ΔT = T_steam − T_boiling) between heating steam and boiling solution — reducing heat transfer rate and requiring larger area

- (C) It makes evaporation impossible for concentrated solutions

- (D) It increases the latent heat of vapourisation

Answer: (B) | If steam is at 120°C and the solution BPR is 5°C, the boiling point of solution = 100°C + 5°C = 105°C (instead of 100°C for pure water). ΔT = 120 − 105 = 15°C (instead of 20°C for pure water) — a 25% reduction in driving force. The same heat exchanger would evaporate 25% less water per unit time. BPR must be accounted for in evaporator sizing.

Q3. The falling film evaporator is preferred over the climbing film evaporator for thermolabile pharmaceutical solutions because:

- (A) Falling film operates at higher temperatures

- (B) Falling film has shorter residence time (< 1 second) — liquid flows rapidly downward by gravity; very short contact with heated surface minimises thermal degradation

- (C) Falling film operates at higher pressure for better heat transfer

- (D) Falling film produces a more concentrated product per pass

Answer: (B) | In falling film, liquid enters at the top and falls as a thin film down the tubes in seconds. The very short residence time (< 1 second per pass) minimises thermal exposure. Combined with vacuum operation (lower boiling temperature), falling film evaporators are the first choice for heat-sensitive pharmaceutical solutions — vitamins, enzymes, herbal bioactives, antibiotics.

Q4. In a triple-effect forward feed evaporator, the most concentrated product exits from:

- (A) Effect 1 — highest temperature for maximum solubility

- (B) Effect 3 — lowest temperature and lowest pressure (highest vacuum)

- (C) The central effect (Effect 2)

- (D) The steam inlet of Effect 1

Answer: (B) | In forward feed, feed enters Effect 1 (highest temperature, lowest vacuum) and flows through Effects 2 and 3 in sequence. Product exits from Effect 3 (lowest temperature, highest vacuum). For thermolabile solutions, this is advantageous — the most concentrated liquid (highest thermal stress at high solids) is handled at the lowest temperature.

Q5. Forced circulation evaporator uses a pump primarily to:

- (A) Create vacuum in the evaporator chamber

- (B) Circulate liquid at high velocity through the external heat exchanger — preventing boiling in the exchanger (under positive pressure) and preventing fouling (high turbulence)

- (C) Compress the vapour produced for reuse as heating steam

- (D) Force feed into the evaporator against gravity

Answer: (B) | The circulation pump creates two important effects: (1) high liquid velocity (1.5–3 m/s) through the heat exchanger maintains turbulent flow — high heat transfer coefficient and prevents fouling by viscous or crystal-forming solutions; (2) positive pressure in the heat exchanger tubes prevents boiling there — boiling occurs only in the flash chamber where pressure is lower. This allows heating of viscous or fouling-prone solutions that would block natural circulation evaporators.

Q6. Vacuum evaporation reduces the boiling point of water to approximately 30°C when vacuum level is:

- (A) 200 mbar

- (B) 100 mbar

- (C) 50 mbar

- (D) 25 mbar

Answer: (D) | Water boiling point vs. pressure: 100 mbar → 46°C; 50 mbar → 33°C; 25 mbar → 21°C; 30 mbar → approximately 30°C. For highly thermolabile pharmaceutical solutions (enzymes, biologics), very deep vacuum (< 30 mbar) is used to keep boiling temperature below 30°C to prevent thermal denaturation or degradation.

Q7. A 10% w/w extract solution is concentrated to 50% w/w. The kg of water evaporated per kg of feed is:

- (A) 0.60 kg/kg

- (B) 0.80 kg/kg

- (C) 0.40 kg/kg

- (D) 0.50 kg/kg

Answer: (B) | Basis: 1 kg feed containing 0.10 kg solids + 0.90 kg water. At 50% solids: total product = 0.10/0.50 = 0.20 kg. Water in product = 0.20 − 0.10 = 0.10 kg. Water evaporated = 0.90 − 0.10 = 0.80 kg per kg feed.

Q8. The central downcomer in a short-tube (calandria) evaporator serves as:

- (A) The steam inlet channel for the tube bundle

- (B) The unheated passage through which liquid descends and recirculates — completing the natural convection circulation loop driven by density difference

- (C) The vapour outlet channel

- (D) The product discharge tube for concentrated liquid

Answer: (B) | In calandria evaporators, the natural convection circulation loop requires: liquid rises in the heated tubes (lower density due to vapour bubbles), vapour separates above, and liquid descends through the unheated central downcomer (higher density, denser than two-phase mixture in tubes). The density difference drives the circulation without any pump — simple, low-maintenance, GMP-compliant.

Q9. The pharmaceutical advantage of the falling film evaporator over a forced circulation evaporator for a vitamin C (ascorbic acid) solution is:

- (A) Falling film can reach higher concentrations per pass

- (B) Falling film has much shorter residence time — vitamin C is highly thermolabile (oxidises and degrades at > 50°C) — falling film minimises thermal exposure while forced circulation has longer residence time in heated equipment

- (C) Falling film requires no steam — operates on hot water only

- (D) Falling film handles higher viscosity than forced circulation

Answer: (B) | Vitamin C (ascorbic acid) is one of the most thermolabile vitamins — significant degradation at 40–60°C. The falling film's sub-second residence time combined with vacuum operation (boiling at 40–50°C) minimises degradation. The forced circulation evaporator, while effective for viscous materials, has longer liquid residence time in hot zones — problematic for vitamin C and other thermolabile nutrients.

Q10. In a multiple-effect evaporator, the vapour produced in Effect 1 is used as:

- (A) Feed for the liquid inlet of Effect 2

- (B) Heating steam for Effect 2 — replacing live steam and dramatically reducing overall steam consumption

- (C) Cooling medium in the condenser downstream

- (D) Compressed refrigerant for vacuum generation

Answer: (B) | The key principle of multiple-effect evaporation: vapour from Effect 1 (at intermediate temperature and pressure) becomes the heating medium in the shell side of Effect 2. Effect 2 operates at lower pressure so its boiling point < vapour temperature from Effect 1 — creating the temperature driving force for heat transfer. This reuse of vapour saves approximately 1 kg steam per effect per kg water evaporated.

Q11. Entrainment separators in evaporators are used to:

- (A) Separate the feed from the product stream

- (B) Remove entrained liquid droplets from the vapour leaving the evaporator — prevents product loss and contamination of the condensate or next effect heating steam

- (C) Separate steam from the condensate in the heating circuit

- (D) Filter the concentrated product before discharge

Answer: (B) | When vapour leaves the boiling liquid surface, it carries fine liquid droplets (entrainment). Without a separator, these droplets would: contaminate the condenser (lost product); reduce condensate quality; or contaminate the next effect's heating steam. Entrainment separators (mesh pads, chevron vanes, cyclone separators) remove these droplets — the liquid falls back into the evaporator and clean vapour exits. Pharmaceutical evaporators require good entrainment separation to prevent product loss.

Q12. Backward feed multiple-effect evaporation is used when:

- (A) The solution is highly thermolabile — cold feed must be protected from heat

- (B) The product is viscous at low temperatures — backward feed keeps the most concentrated (most viscous) solution at the highest temperature where viscosity is lowest — better pumpability and heat transfer

- (C) Economy is not a priority — backward feed wastes steam

- (D) The feed is dilute and requires minimal concentration

Answer: (B) | In backward feed, feed enters at the lowest temperature effect (Effect 3, highest vacuum) and moves to progressively higher temperature effects, leaving as concentrated product at Effect 1 (highest temperature). This is beneficial when: the product is very viscous at low temperature (would not flow at 30°C but flows well at 80°C); when high concentration is needed and low-temperature handling would cause crystallisation; when feed is cold (avoids heating losses).

Q13. The heat transfer equation Q = U × A × ΔT for evaporators shows that fouling reduces evaporation rate because:

- (A) Fouling increases ΔT between steam and solution

- (B) Fouling deposits on heat exchanger surfaces reduce U (overall heat transfer coefficient) — fouling acts as an additional thermal resistance

- (C) Fouling increases the evaporator economy

- (D) Fouling increases the area A available for heat transfer

Answer: (B) | Fouling deposits (scale, protein, starch, crystalline salts) on heat exchanger tube surfaces add an additional thermal resistance layer. The overall heat transfer coefficient U = 1/(1/h_i + R_fouling + wall resistance + 1/h_o) — R_fouling increases the denominator → U decreases. With the same steam pressure and area, Q = U × A × ΔT decreases → lower evaporation rate. Regular cleaning (CIP) maintains U at design values.

Q14. Why is steam pressure in the FIRST effect of a multiple-effect evaporator higher than in subsequent effects?

- (A) More steam is needed in Effect 1 because feed is coldest there

- (B) Each effect operates at progressively lower pressure (higher vacuum) to ensure that vapour from the previous effect is at a higher temperature than the boiling solution in the next effect — creating the temperature driving force for heat transfer

- (C) High pressure in Effect 1 ensures maximum product concentration

- (D) Effect 1 processes more liquid than subsequent effects

Answer: (B) | For heat transfer to occur in Effect 2, the heating medium (vapour from Effect 1) must be hotter than the boiling solution in Effect 2. This requires Effect 2 to operate at lower pressure (lower boiling point) than Effect 1. The pressure cascade: Effect 1 (highest, ~atmospheric) → Effect 2 (intermediate) → Effect 3 (lowest, maximum vacuum). Each effect's boiling point must be below the vapour temperature from the previous effect.

Q15. In pharmaceutical herbal extract concentration, the primary reason for choosing vacuum evaporation over atmospheric evaporation is:

- (A) Vacuum evaporation is faster than atmospheric

- (B) Vacuum evaporation reduces the boiling point — enabling concentration at 40–60°C instead of 100°C — preventing thermal degradation of heat-sensitive bioactive compounds

- (C) Vacuum evaporation requires less heat than atmospheric

- (D) Vacuum equipment is cheaper than atmospheric evaporators

Answer: (B) | Herbal extracts contain thermolabile bioactives — flavonoids, alkaloids, terpenes, glycosides, vitamins — that degrade, isomerise, or oxidise at 100°C (atmospheric boiling point). At 100 mbar vacuum, water boils at ~46°C — these compounds are stable at 46°C. Vacuum evaporation preserves pharmacological potency and quality of herbal medicines. This is why rotary evaporators (laboratory scale) and falling film vacuum evaporators (industrial scale) are standard in herbal pharmaceutical manufacturing.

## Short Answer Questions — Chapter 8

Q1. Define evaporation. Distinguish it from drying and distillation. State five pharmaceutical applications of evaporation.

Q2. Explain Boiling Point Rise (BPR) and Economy with formulas. How does multiple-effect evaporation improve economy?

Q3. Compare climbing film and falling film evaporators — principle, liquid flow direction, residence time, suitability for thermolabile solutions.

Q4. Describe the forced circulation evaporator — construction, working, advantages for viscous solutions.

Q5. A 2000 kg/hr feed of 8% w/w solution is concentrated to 40% w/w. Calculate water evaporated per hour and the % concentration factor.

## Long Essay Questions — Chapter 8

Q1. Describe the theory of evaporation. Include: definition and pharmaceutical objectives; Boiling Point Rise; Economy (single and multiple effect); heat transfer equation Q = U × A × ΔT. Describe short-tube (calandria), climbing film, and falling film evaporators with construction, working, advantages, disadvantages, and pharmaceutical applications.

Q2. Write a comprehensive note on advanced evaporation systems. Describe: forced circulation evaporator; multiple effect evaporation (triple effect — forward and backward feed, economy calculation); vacuum evaporation (importance for thermolabile drugs, vacuum levels vs. boiling points). Present a complete worked example on economy and evaporation rate calculation.